# Week 2 – Networking Fundamentals for DevOps

## Slide 1: Introduction to Networking

Networking means connecting devices so they can communicate.

Examples of devices:

- Computer

- Server

- Router

- Switch

- Mobile phone

Network allows:

- Sharing data

- Internet access

- Communication between systems

## Slide 2: Types of Networks

### LAN (Local Area Network)

- Small area network

- Example: Office, home, lab

### WAN (Wide Area Network)

- Large area network

- Example: Internet

### MAN (Metropolitan Area Network)

- Covers a city

### PAN (Personal Area Network)

- Covers a few meters

- Example: Bluetooth connection

## Slide 3: Network Devices

### Router

- Connects different networks

- Sends data between networks

### Switch

- Connects devices inside same network

### Hub

- Sends data to every device

### Modem

- Connects network to internet

### Firewall

- Protects network from unauthorized access

## Slide 4: IP Address

IP Address identifies a device on a network.

Two types:

- IPv4

- IPv6

Example IPv4:

- 192.168.1.10

IPv4 contains 4 numbers separated by dots. Each number ranges from 0 to 255.

## Slide 5: Private and Public IP Address

### Private IP

Used inside a local network. Examples:

- 192.168.x.x

- 10.x.x.x

- 172.16.x.x to 172.31.x.x

### Public IP

Used on the internet. Assigned by ISP.

Example: Your home router has a public IP, while your laptop has a private IP.

## Slide 6: Static vs Dynamic IP

### Static IP

- Never changes

- Used for servers

### Dynamic IP

- Changes automatically

- Assigned by DHCP

## Slide 7: Subnet Mask

Subnet mask tells which part of the IP address belongs to network and which part belongs to host.

Example: IP Address: 192.168.1.10 Subnet Mask: 255.255.255.0

This means:

- Network = 192.168.1

- Host = 10

## Slide 8: Default Gateway

Gateway is the device used to leave the local network. Usually the router IP.

Example:

- Computer IP = 192.168.1.10

- Gateway = 192.168.1.1

## Slide 9: DNS (Domain Name System)

DNS converts domain names into IP addresses.

Example:

- google.com → 142.250.x.x

Without DNS, we would have to remember IP addresses.

## Slide 10: TCP/IP Model

TCP/IP model has 4 layers:

- Application Layer

- Transport Layer

- Internet Layer

- Network Access Layer

### Application Layer

Protocols:

- HTTP

- HTTPS

- FTP

- SSH

### Transport Layer

Protocols:

- TCP

- UDP

### Internet Layer

- IP

- ICMP

### Network Access Layer

- Ethernet

- Wi-Fi

## Slide 11: TCP vs UDP

### TCP

- Reliable

- Connection-oriented

- Slower

Used in:

- Web browsing

- Email

- File transfer

### UDP

- Faster

- No guarantee of delivery

- Connectionless

Used in:

- Video streaming

- Gaming

- Live calls

## Slide 12: Common Network Ports

| Service | Port |
| --- | --- |
| HTTP | 80 |
| HTTPS | 443 |
| SSH | 22 |
| FTP | 21 |
| DNS | 53 |
| SMTP | 25 |

## Slide 13: Important Protocols

### HTTP

Used for websites.

### HTTPS

Secure version of HTTP.

### FTP

Used to transfer files.

### SSH

Used to connect to remote server securely.

### SMTP

Used to send emails.

### ICMP

Used for ping command.

## Slide 14: Basic Networking Commands in Linux

### ping

Checks connectivity. Example: ping google.com

### ifconfig / ip addr

Shows IP address.

### traceroute

Shows path to destination.

### netstat

Shows active connections.

### nslookup

Checks DNS records.

## Slide 15: What is DHCP?

DHCP automatically gives IP address to devices.

Without DHCP:

- We must manually configure every device

With DHCP:

- Device automatically receives:

  - IP address

  - Gateway

  - DNS

## Slide 16: Network Security Basics

Important concepts:

- Firewall

- Antivirus

- VPN

- Strong passwords

- SSH instead of Telnet

## Slide 17: Networking in DevOps

Networking is important in DevOps because:

- Servers communicate through networks

- Applications connect to databases

- Docker and Kubernetes use networking

- Cloud platforms require networking knowledge

Examples:

- Accessing EC2 server using SSH

- Opening port 8080 for web application

- Configuring security groups in AWS

## Slide 18: Practical Lab Ideas

### Lab 1

Find your computer IP address.

### Lab 2

Ping google.com and observe response.

### Lab 3

Use nslookup to find IP of google.com.

### Lab 4

Use traceroute to check path.

### Lab 5

Connect to Linux server using SSH.

## Slide 19: Summary of Week 2

After week 2, students should know:

- Network basics

- IP addressing

- DNS

- TCP/IP model

- Network protocols and ports

- Basic Linux networking commands

- How networking is used in DevOps